MINPULS 4 CNC electrical discharge machining

Machine tool specifications

| Machine tool: | MINPULS 4 |
| --- | --- |
| Shaft travel(X×Y×Z) | 450×350×300 mm |
| Internal dimensions of the processing fluid tank (width × depth × height) | 970×750×390 mm |
| Maximum processing fluid height | 280 mm |
| Workbench dimensions (width × depth) | 650×500 mm |
| Fastest movement speed | 5000 mm/min |
| Fastest jump speed along the Z-axis | 10000 mm/min |
| Oil tank lifting system | Vertical lifting |
| Maximum electrode weight (standard spindle head) | 30 kg |
| Maximum workpiece weight | 500 kg |
| The lowest point of the Z-axis is the standard spindle head (below the chuck to the worktable). | 250 mm |
| Table height | 830 mm |
| Workbench T-slots (width × number of slots) | 10 mm×5 slots |
| Machine tool dimensions (width × depth × height) | 1800×2200×2350 mm |
| Machine tool weight (including power supply frame) | 3100 kg |
| Processing fluid supply device |  |
| Type | Split-type machine tool |
| Total amount of processing fluid | 400L |
| Fast oil pump | 1.1 kW |
| Cooling pump | 0.75 kW |
| Processing fluid filtration | External pressure paper filter element |
| Filter element root | 2 |
| Power supply device |  |
| Type | Split-type machine tool |
| Main panel type | Integrated electrical control box (optional hanging type) |
| Portable control panel | Intelligent multi-functional |
| Machine power supply | 60A (80A optional) |
| Accuracy |  |
| Positioning accuracy (full stroke) | ±0.005 mm |
| Repeatability (full stroke) | ±0.003 mm |

Standard configuration

| Machine tool hardware |  |
| --- | --- |
| Control system | MINPULS |
| Three-axis lead screw guide rail | PMI/THK |
| Three-axis servo motor | Mitsubishi fiber optic servo |
| Ultrafine circuit |  |
| Automatic fire extinguisher |  |
| Extinguishing agent capacity | 1.5L |
| Number of sensors | 1pc |
| Number of nozzles | 1pc |
| External dimensions (width × depth × height) | 210×167×405mm |
| Fire extinguishing agent | Contains a 6% light aqueous solution (foam extinguishing agent). |
| Detector type | Bimetallic |
| Operating temperature | 60℃ |
| Power supply | AC200V |

Working fluid cooling device

| Standard cooling energy | 50Hz3720W |
| --- | --- |
| Ambient temperature top line | 5~40℃ |
| Oil temperature sensor | Resistance Temperature Detector |
| Electric rotary circulating pump | 500W(2P) |
| External dimensions (width × depth × height) | 360×420×755mm |
| Weight | 45kg |
| Portable control panel |  |
| Power circuit breaker |  |